there has a little error at the thrid paragraph's begin:

"Figure 23.7.2" -> "Figure 23.8.2"

Paper Review：An 88.36TOPS/W Bit-Level- Weight-Compressed LLM Accelerator with Cluster-Aligned INT-FP-GEMM and Bi-Dimensional Workﬂow Reformulation

Conference: ISSCC 2025 February 16-20

Summary

This paper addresses three major challenges in eﬃcient deployment of large language models (LLMs) by proposing a novel accelerator architecture. The key

innovations include:

ABAF (Active-Bit-Allocate Format): The paper introduces a compression format for weights and KV-cache, along with a Prime-Number-Based Calculator (PNBC) for fast decoding without requiring a lookup table. This design eﬀectively reduces the data transfer and storage overhead caused by frequent EMA access during LLM inference.

- CAMP-PE (Cluster-Aligned Mixed-Precision Processing Element): To address the low energy eﬃciency of mixed-precision GEMM operations, the authors propose a clustering approach that converts ﬂoating-point operations into integer computations by grouping activations by exponent, signiﬁcantly improving eﬃciency.

- BMSR (Bi-Dimensional MAC-based SFU Reformulation): To reduce SFU bottlenecks in nonlinear operations, the paper reformulates nonlinear functions as MAC operations, allowing them to be processed directly within PEs and increasing overall hardware utilization.

Strengths

ABAF presents an elegant integration of compression, decoding, and dataﬂow reformation, eﬀectively addressing the high EMA cost in autoregressive models.

CAMP-PE simpliﬁes GEMM operations by transforming activation computations into integer operations using clustering.

BMSR smartly converts nonlinear functions into linear MACs, allowing these operations to be executed in PEs and bypassing SFU limitations.

Optimization targeting LLM model deployment, addressing a key issue in current AI development.

Strong persuasiveness from design concept to actual testing.

Circuit architecture is problem-oriented with clear objectives.

Weaknesses

The paper lacks suﬃcient evidence to support the claim that AWSC (Active Word Shift Compression) does not aﬀect model accuracy. Bit-shifting operations in deep learning can introduce rounding or truncation errors. The

paper does not specify how many bits are used for intermediate values or whether saturation/clipping mechanisms are applied.

The use of Taylor expansion and Horner’s method for nonlinear function

approximation lacks detailed error analysis.

The paper does not discuss adaptability to bit-widths other than 8 bits, raising concerns about generality.

Evaluation on larger models such as GPT-3 or LLaMA-13B is absent. It is unclear whether the architecture can maintain accuracy or avoid performance bottlenecks like I/O bandwidth or increasing EMA with longer context lengths.

Suggestions for Improvement

Include precision error analysis and evaluate the accelerator's scalability under higher sparsity.

Provide performance results on larger-scale LLMs to assess generalization and robustness.

Overall Evaluation

This paper presents a strong hardware-software co-design addressing real-world bottlenecks in LLM inference. With additional precision analysis and generality validation, it has the potential to become a valuable reference for both edge and cloud LLM accelerator designs.